Quick Start guide

DCC ESP with Railcom POM-read

Thank you for your purchase.  This board is an updated version of the Instructables project found here  https://www.instructables.com/member/Computski/

Specifically, this project https://www.instructables.com/ESP8266-DCC-Controller/

For support you can ask questions on the Instructables site or email me at computski@gmail.com

CAUTION:  Please double check you have the correct polarity when connecting up the 12V DC supply.  The board is not reverse polarity protected and you could damage it.

Your board is fully built, programmed and tested.  Just connect 12v DC power and X-Y connections to the track and you are ready to run DCC locomotives via Engine Driver on an Android phone.

The board is capable of supporting an LCD display, 16 key keypad and a jogwheel acting as an optional local hardware interface.  See the Instructable for more details.

The board can handle a max of 4 Amps, however, the current trip threshold is set to 1 Amp by default.  You can change this through the hardware screen on the web interface, or through the hardware screen under the DCC ESP settings found in the Engine Driver app on your phone.

You can also feed the unit with a higher DC voltage, say 14V if you want a higher track voltage, but note the controller has an over-voltage trip which is set to 15V by default.  You can change this in the hardware screen.

I also enclose a revised user guide, and can email you a copy of this on request.

Happy railroading!